Report Book for Smart Home using IoT

Prepared by:
Khaled Iyad Al birouty

Supervised by:
Dr. Samer Hussam Eddin

ENG: Shadi saab

Academic Year:
2025 – 2026

Chapter 1: General Introduction

1-1- Project Overview and Background:

The rapid convergence of embedded computing and the Internet of Things (IoT) has fundamentally

transformed home automation from a luxury convenience into an indispensable framework for

safeguarding human lives and residential infrastructure. This graduation project focuses on the

architectural design, implementation, and empirical verification of a comprehensive, decentralized

smart home platform uniting environmental hazard mitigation, physical access security, visual

surveillance, and remote load automation. The engineered system leverages a distributed edge-

computing paradigm utilizing dual ESP32 microcontrollers. These dedicated nodes interface

seamlessly with a lightweight cloud MQTT broker, delivering deterministic sub-second emergency

response times without the overhead or expense of centralized local computers or complex

database engines.

1-2- Problem Statement and Significance:

Domestic gas leakages and thermal fires represent critical emergencies requiring rapid mechanical

suppression within fractions of a second—a task inherently difficult for humans during sleep or

absence. Furthermore, current academic prototypes and commercial smart home products suffer

from three pervasive engineering shortcomings:

1. Excessive Network Dependency: Centralized architectures rely on active internet connections or

continuous server pinging, rendering life-safety mechanisms completely inoperable during

connectivity dropouts.

2. Functional Fragmentation: Fire detection, electronic door security, and video streaming typically

operate as disconnected silos, preventing synergistic, incident-triggered actions (such as focusing a

camera feed directly on a gas breach).

3. Prohibitive Deployment Cost &amp; Database Complexity: Conventional designs rely on costly single-

board computers (such as Raspberry Pi 4) and relational database servers, introducing high idle

power consumption, maintenance bottlenecks, and single-point system failures.

The significance of this project lies in providing an autonomous, highly reliable, and cost-effective

edge system that ensures instantaneous life-safety intervention regardless of network availability.

1-3- Project Objectives:

• Main Objective:

To design and deploy a low-cost, fully integrated, distributed IoT smart home system integrating

environmental safety, two-factor physical access control, video monitoring, and remote automation

based on dual ESP32 microcontrollers and cloud MQTT communications without local server or

database overhead.

• Specific Objectives and Technical Requirements:

1. Autonomous Hazard Mitigation: Detect combustible gas leaks (MQ-2) and abnormal temperature

surges (DHT22) to autonomously trigger an exhaust fan and water mist suppression pump locally

in ≤ 250 ms.

2. Multi-Factor Access Security: Implement a two-factor authentication (2FA) entrance mechanism

utilizing 13.56 MHz RFID (MFRC522) and a 4x4 matrix keypad to control a  solenoid lock.

3. Perimeter Intrusion Detection: Monitor physical door contact using a magnetic reed switch to

trigger immediate local alarms and cloud incident dispatches upon unauthorized forced entry.

4. Integrated Video Surveillance: Interface an ESP32-CAM module to provide local MJPEG video

streaming and transmit real-time snapshot captures upon safety or security alarm activations.

5. Responsive Cloud Dashboard: Develop a secure, low-latency web dashboard enabling worldwide

telemetry monitoring and manual appliance switching via MQTT over WebSockets.

1-4- Definitions and Key Concepts:

- Internet of Things (IoT): An interconnected global infrastructure of uniquely addressable physical items featuring embedded sensing, computing, and actuation,enabling seamless data exchange across communication networks

- Edge Computing: A distributed computing architecture where data processing, condition analysis, and control decisions occur locally near the data source (edge devices) rather than within centralized data centers, drastically slashing latency and network traffic

- Edge Computing: A distributed computing architecture where data processing, condition analysis, and control decisions occur locally near the data source (edge devices) rather than within centralized data centers, drastically slashing latency and network traffic

- Edge Computing: A distributed computing architecture where data processing, condition analysis, and control decisions occur locally near the data source (edge devices) rather than within centralized data centers, drastically slashing latency and network traffic

1-4- Definitions and Key Concepts:

- Scope Boundaries:

Firmware development for ESP32 and ESP32-CAM (C++/FreeRTOS), hardware power distribution (12V/5V), sensor signal acquisition, local edge actuation, cloud MQTT publish/subscribe messaging, and web dashboard hosting.

- Technical Specifications:

- Actuation Latency: Hardware relay switching execution within ≤ 250 ms of sensor threshold violation.

- Fail-Safe Autonomy: 100% operational continuity of safety venting and RFID door access during Wi-Fi or internet outages.

-  Physical Lock Security: Fail-Secure solenoid configuration retaining locked status during power

failure, with manual mechanical key override.

-  Power Efficiency: Total idle system power consumption bounded below .

Chapter 2: Review of Literature and Previous Studies

2-1- Previous Studies:

Recent scholarly literature (2023–2025) reflects intensive research into smart home automation

and embedded safety. A rigorous evaluation was performed on three primary open-access

benchmark studies:

1. Fire and Combustible Gas Mitigation System (Rahman et al., 2023):

The authors developed an embedded safety prototype utilizing an Arduino Uno interfaced with an

external ESP8266 Wi-Fi shield. The system monitors MQ-2 and DHT11 sensor readings,

autonomously driving an exhaust fan and solenoid spray valve during hazard events, while

transmitting data to ThingSpeak.

• Key Concepts Utilized: Direct hardware relay switching upon passing calibrated environmental

thresholds.

• Identified Limitations: Single-purpose architecture devoid of physical entrance access or visual

monitoring, combined with the severe bandwidth overhead of HTTP polling.

2. Multi-Factor Smart Access Control Architecture (Kumar &amp; Patel, 2023):

This study presented an electronic door security solution centered on an ESP32 microcontroller,

integrating an MFRC522 RFID reader, a 4x4 keypad, and a magnetic reed switch to prevent

unauthorized physical intrusions.

• Key Concepts Utilized: Hybrid two-factor credential validation and interrupt-driven perimeter

breach sensing.

• Identified Limitations: Complete isolation from environmental safety systems, absence of visual

streaming, and lack of unified remote home automation.

3. Low-Latency Surveillance and Control Infrastructure (Zhang &amp; Wei, 2024):

Zhang and Wei designed a smart home network powered by a Raspberry Pi 4 functioning as a

central gateway, hosting Mosquitto MQTT, a Node.js server, WebSockets, and RTSP IP camera

streaming to achieve high-speed responsiveness.

• Key Concepts Utilized: Low-latency MQTT publish-subscribe messaging and dynamic multimedia

transport.

• Identified Limitations: High deployment cost, vulnerability to single-point server failure, and total

omission of embedded physical actuators and door locks.

2-2- Comparative Analysis of Existing Systems:

| Evaluation Metric | Safety Study (2023) | Access Study (2023) | Network Study (2024) | Our Proposed System |
| --- | --- | --- | --- | --- |
| Primary Domain & Scope | Gas venting & fire suppression | Physical door access control | Data transport & surveillance | Comprehensive (Safety + Security + Video + IoT) |
| Core Processing Unit | Arduino Uno + ESP8266 | Single ESP32 MCU | Raspberry Pi 4 (SBC) | Dual Distributed ESP32 + ESP32-CAM |
| Computing Paradigm | Isolated Primitive Edge | Standalone Local Logic | Centralized Local Server | Autonomous Edge Logic + Cloud Web Telemetry |
| Transport Protocol | HTTP Polling (High Overhead) | HTTP Webhooks | Local MQTT + WebSockets | Lightweight Cloud MQTT (QoS-driven) |
| Visual Surveillance | Not Supported | Not Supported | RTSP IP Camera Streams | ESP32-CAM MJPEG + Incident Captures |
| Hardware Cost & Complexity | Very Low / Limited Function | Low / Single-purpose | Prohibitive (SBC & Peripherals) | Highly Cost-Optimized & Scalable |
| Offline Blackout Resilience | Operates Locally | Operates Locally | Total System Freeze | 100% Operational Continuity for Life-Safety |

2-3- Research Gap and Proposed Innovation:

The comparative analysis reveals an unresolved polarization in contemporary IoT research: lightweight microcontroller setups lack cohesive multimedia and networking integration, while sophisticated IoT gateways rely on expensive single-board computers prone to catastrophic central failures. Our proposed engineering innovation resolves these deficiencies through:
1. Distributed Edge Partitioning: Allocating time-critical hazard handling to discrete, inexpensive ESP32 microcontrollers eliminates server bottlenecks and guarantees life-safety execution during network collapse.
2. Cohesive Cross-Domain Interplay: Unifying environmental sensing and physical access control with an ESP32-CAM node, turning hazardous gas readings or door breaches into instantaneous visual alerts.
3. Zero-Database Cloud Connectivity: Replacing complex relational databases with FreeRTOS state tables and cloud-hosted MQTT topics, ensuring ultra-low latency, zero maintenance, and worldwide dashboard control.

2-4- Theoretical framework

• Theoretical Framework & Control Methodologies:
The system is formulated upon Closed-Loop Event-Driven Control Theory. Environmental sensor feedback is continuously sampled and mapped against calibrated mathematical hazard thresholds. When a state boundary is violated, deterministic interrupt service routines immediately generate actuation signals via the edge processor.

• Electrical, Electronic, and Mechanical Hardware Components:
1. Compute Nodes: Two ESP32 DevKit V1 microcontrollers (Xtensa Dual-Core 240 MHz, 520 KB SRAM) and one ESP32-CAM module (OV2640 sensor).
2. Sensing Devices: MQ-2 metal-oxide gas sensor, DHT22 capacitive temperature/humidity sensor, MFRC522 13.56 MHz RFID reader, and a magnetic reed switch.
3. Actuators & Outputs: 12V Solenoid door lock, 12V brushless exhaust fan, 12V submersible water pump, 5V active piezo buzzer, 4x4 matrix keypad, and optocoupler-isolated relay modules.
4. Power Distribution Infrastructure: 12V / 5A DC primary power supply, LM2596 step-down DC-DC buck converters (5V / 3A rail), common system ground, and flyback protection diodes.

• Supporting Software and Engineering Tools:
1. VS Code with PlatformIO / Arduino IDE for C++ embedded firmware engineering.
2. HiveMQ Cloud / EMQX broker for enterprise-grade cloud MQTT messaging.
3. Proteus and Fritzing for electrical schematic verification and PCB layout routing.
4. HTML5, CSS3, and JavaScript (WebSocket client) for cross-platform cloud dashboard development.

2-5- Chapter conclusion:

This chapter presented an extensive review of recent literature in smart home automation, offering a critical comparative evaluation that clearly exposed the research gaps addressed by this work. Furthermore, the complete theoretical, electrical, and software frameworks underpinning the platform were rigorously articulated. The subsequent chapter (Chapter 3) details the technical specifications, system block diagrams, power rail calculations, and hardware pin mappings required to implement this architecture.